# F-16 18 GHz HIRF Training

EMA3D Primer for Creating a Basic HIRF Problem on an F-16 Cockpit

Prepared by Sean Rozowski

December 2018

# Contents

Contents	i

Problem Overview	1

Opening the File	1

Viewing the Model	1

Defining the Model Units	2

Defining the Simulation Domain	2

Meshing the Geometry	3

Defining the Simulation Time Steps	3

Defining Background Properties	3

Defining Material Properties	3

Assigning Material Properties	4

Defining the Plane Wave Source	5

Defining Boundary Conditions	5

Creating Probes in EMA3D	5

Creating the EMA3D Input File	6

Running the Integrated Simulation	6

Run EMA3D Simulation in Parallel	6

Analyzing Results	8

Python Analysis	8

MATLAB Analysis	8

## Problem Overview

This tutorial will introduce the user to the EMA3D workflow for a HIRF simulation. This workflow includes setting up the problem space, assigning properties and defining probes. This training module assumes that the user has a basic understanding of the CADfix and EMA3D GUI. It is recommended to see other trainings as well to help develop stronger understanding of CADfix and EMA3D.

A pre-simplified F-16 cockpit model will be used to simulate this problem. The F-16 cockpit will be exposed to a plane wave of a gaussian pulse to simulate multiple frequencies at once. This will allow the canopy shielding effectiveness to be measured.

## Opening the File

- Under the Windows start menu launch CADfix for EMA3D

- Click on the Open Model button.

- Select CADfix GDX as the File type

- Import the file titled “F-16_18GHz_Demo.gdx.”

## Viewing the Model

The imported model will automatically plot the line connectivity. To see the entire model the surfaces will need to be plotted.

- Copy and paste the following into the CADfix command line:

PLOT SI ALL ASGD

PLOC SHADE BOTH

REP

NOTE: More detailed information about using the CADfix command line usage and manipulating CADfix geometry can be found in the Introduction to EMA3D and CADfix Tutorial.

The model is currently in an assembly and needs to be broken up into CADfix sets.

- Enter the following into the command line to create the needed sets:

SETA SKIN Z2

SETA CANOPY Z4

SETA BOXES Z6

SETA SEAM Z8

SETA PINS Z10

SETA GRID1 Z12

SETA GRID2 Z14

SETA GRID3 Z16

## Defining the Model Units

We first need to define the model units for our problem.

- Open the General tools list in the EMA3D tab and click Specify Units

- Select Millimeters in the pop-up window

- Click OK

## Defining the Simulation Domain

Now that our geometry has been defined. It is time to define the computational domain or Lattice.

- Click Define Lattice in the General tools list of the EMA3D tab

- Click on the Lattice Parameters tool in the expanded section

  - Define the X-Lattice to be 2210 to 4240

  - Define the Y-Lattice to be -870 to 870

  - Define the Z-Lattice to be 1960 to 3500

- Change the cell size to 1.6 for the x, y, and z directions.

Ensure the parameters are entered correctly as seen below.

- Click Apply

- The boundary of the problem space can be seen by copying the following into the command line:

PLUS LATE

## Meshing the Geometry

Next, we need to mesh the geometry.

- Click Mesh Geometry in the General tools list of the EMA3D tab to expand the section

- Click Setup

- Click Mesh

## Defining the Simulation Time Steps

Next the simulation time steps must be defined. Our simulation will run to 0.02 μs with a time increment of 1.6e-12 s.

- Click Define Time Step in the EMA tab

- Click Time Step

- Enter 1.6e-12 into the box labeled Time step

- Type 2.0e-8 in the box labeled Final Simulation Time

- Click Apply

## Defining Background Properties

Next, we need to define the property of empty space in the problem.

- Click Define Background

The default values are for air which will be used for this problem.

- Click Ok

## Defining Material Properties

The geometry in the model will need properties assigned to it in order to simulate the problem. The entire model is composed of surfaces, so isotropic surface materials will be defined along with a PEC line material.

- Click Define Properties in the EMA tab

- Click New in the “EMA3D Property Editor” window

- Enter BOXMAT into the Name field

- In the Type spin box, change the definition to Isotropic Body

- Click Ok

- In the popup window, enter 1.0e-2 in the Electric Conductivity field

- Leave the other fields as the default and click OK

Repeat Steps 2-7 with the names, material types and electric conductivities from the following table:

| EMA3D Material Name | EMA3D Material Type | Electric Conductivity |
| --- | --- | --- |
| SKINMAT | Isotropic Surface | 6.85E+06 |
| SEAMMAT | Isotropic Surface | 0 |
| CANOPYMAT | Isotropic Surface | 1.85E+01 |
| PIN | PEC Line | N/A |

## Assigning Material Properties

Now that we have our properties defined, we can assign the properties to the geometry itself.

- Select BOXMAT in the “Property Editor” popup window

- Click Assign

- Type BOXES into the field labeled CADfix Part

- Click Apply

- Select PIN as the EMA3D Property

- Type PINS as the CADfix Part

- Click Apply

- Select CANOPYMAT as the EMA3D Property

- Type CANOPY as the CADfix Part

- Click Apply

- Select SEAMMAT as the EMA3D Property

- Type SEAM as the CADfix Part

- Click Apply

- Select SKINMAT as the EMA3D Property

- Type SKIN as the CADfix Part

- Click Ok

## Defining the Plane Wave Source

Next the HIRF plane wave source will need to be defined.

- Click Plane Wave in the Define Sources section of the EMA tab

- Enter gauss.dat for the Source File

- Click Auto-Place for the Huygen’s Surface

- For the Propagation values enter the following:

    - Θ = 90

    - Φ = 90

    - α = 0

    - β = 0

- Click Apply

## Defining Boundary Conditions

Now we can define the boundary conditions for the problem. For HIRF, the perfectly matched layer boundary condition will be used for all boundaries.

- Click Define Boundary Conditions in the EMA tab

- Select PML for all boundaries

- Enter 32 for the # Layers and press Apply

## Creating Probes in EMA3D

Next, we’ll create the electric field probes that will capture the fields inside the canopy and be used to compute the shielding effectiveness.

- Click Define Probes in the EMA tab

- Right Click on the New Probe and select New

- Enter GRID1 as the Set containing points

- For the Probe Points enter the set GRID1 and press Enter

- Select positive Ex, Ey and Ez fields as the outputs

- The Output Types needed are Numerical and Time

- Enter 1.6e-12 into the box labeled Step Time

- Type 2.0e-8 in the box labeled Stop Time

- Click Apply

Repeat steps 2-9 two more times, entering GRID2 and GRID3 as the Set containing points.

## Creating the EMA3D Input File

We now need to generate the EMA3D input file which is required for EMA3D simulations.

- In the EMA3D tab navigate to the General tools list

- Click Review Tool to expand the toolbox

- Click Review

- Click Create EMA3D Input File

- Click OK to generate the .emin file

NOTE: Due to the relatively small mesh size in the model it may take a while to create the input file.

## Running the Integrated Simulation

We are finally prepared to run the EMA3D simulation.

- Navigate to the General tools list under the EMA3D tab

- Click Run EMA3D to launch the EMA3D Run Tool. This may take several seconds to launch

- Select the Run New EMA3D Simulation radio button

- In the Entry titled EMA3D Input File click the Browse button

- Select the .emin file we created earlier

- Click Run EMA3D to start the simulation

NOTE: The simulation will work in the EMA3D run tool, but it will take multiple days to finish the simulation. It is suggested to run this simulation in parallel if at all possible.

## Run EMA3D Simulation in Parallel

EMA3D allows for the simulation to be parallelized across multiple computing processors. This can dramatically speed up simulation time and is suggested should multiple computing cores be available for use.

- To check how many cores are available on any compute, open the task manager and click on the performance tab and select the tab labeled CPU. The total amount of logical processors can be found in the bottom right corner.

- The computer this demo was run on has 88 processors available, but for this example 32 processors will be used.

- Open the file named F-16_18GHz_Demo.emin in any text editor.

- Edit the line under !MPI BLOCKS from 1 1 1  to an even distribution of the number of processors you are going to run this on. For example, since this computer will run the problem on 32 cores the line will be changed to 4 4 2. It is important to note that this distribution will most likely not be the fastest way to solve this problem. The fastest way to solve the problem can be found by running the first 15 – 20 time steps of the problem using different distributions in the !MPI BLOCKS section.

- Open a command prompt window and navigate to the folder you were working out of.

- Type mpiexec.exe -n # ema3d F-16_18GHz_Demo.emin where # is the number of cores you are running the problem on, in the case of the computer the example was run on this would be 32.

- Hit Enter and the program should begin.

NOTE: If this is your first time running MPI the program will ask for an account user name and password, this is just the user name and password for the computer you are using. Once that information is entered the program should begin.

- EMA3D will check for issues in the input file and then prompt you if you would like to start the sim. Type Y and hit Enter and the program will begin to run the simulation.

## Analyzing Results

The data that is captured in the simulation can be looked at in using the EMA3D Run tool. However, the statistical analysis for the shielding effectiveness of the canopy must be done in an outside program. Two scripts have been prepared to do this analysis, one in python and one in MATLAB. Please select whichever script you prefer.

### Python Analysis

- Open the file named F-16_18GHz_demo.py.

- Scroll down to line 110 and change di to the directory containing the data.

- Press the Run  at the top of the screen and allow a couple of minutes for the simulation to run.

### MATLAB Analysis

- Open the file named F_16_18GHz_Demo.m.

- Ensure the data is in the same directory as the script and press Run.